Sassafrassss — Cutover Day Plan

UniFi EX7  ·  Parallel Test → Cutover → Burn-in

April 2026  ·  West Palm Beach FL

Overview

| Phase | What Happens |
| --- | --- |
| Phase 0 — Prep | Back up everything. Rename AT&T SSID to ATT_OLD. Confirm EX7 WAN is getting IP from AT&T. |
| Phase 1 — Parallel Test | Both routers running. Devices connect to EX7. Set all overrides and static IPs. Verify everything works before cutting over. |
| Phase 2 — Cutover | Enable AT&T IP Passthrough. EX7 takes over WAN. Start 24hr burn-in. |
| Phase 3 — Post Burn-in | Audit clients. Run 21 verification tests. Hide SSID. Decommission AT&T. Final backup. |

⚠ NOTE: Do not enable IP Passthrough until ALL Phase 1 steps are complete and verified.

⚠ NOTE: Keep AT&T router physically connected and powered until Phase 3 is complete — it is your rollback.

# Phase 0 — Before You Start

Color key: Green = verified OK · Blue = device config · Red = critical · Yellow = caution

| Step | Action | Detail | ✓ |
| --- | --- | --- | --- |
| 0.1 | Download runbook v5 | Upload sassafrassss_runbook_v5.docx to Claude if you need help during the process. | ☐ |
| 0.2 | Take AT&T backup | Log into AT&T router → note all connected device MACs and IPs for reference. | ☐ |
| 0.3 | Take EX7 backup | Settings → System → Backup → Download. Save to Google Drive labeled pre-cutover-test.unf | ☐ |
| 0.4 | Rename AT&T SSID | Log into AT&T router → rename WiFi SSID to ATT_OLD (2.4 and 5GHz). This prevents devices bouncing between routers. | ☐ |
| 0.5 | Confirm EX7 WAN IP | EX7 → Settings → Internet → confirm WAN shows a 192.168.x.x IP from AT&T DHCP. If blank — check ethernet cable from AT&T LAN port to EX7 WAN port. | ☐ |

# Phase 1 — Parallel Testing

AT&T router stays ON throughout Phase 1. Double NAT is acceptable for testing. Internet works normally.

| Step | Action | Detail | ✓ |
| --- | --- | --- | --- |
| 1.1 | Power on EX7 | EX7 goes live. AT&T stays ON. Both running simultaneously. Double NAT is fine for testing. | ☐ |
| 1.2 | Connect your iPhone to Sassafrassss | Confirm you get 192.168.10.x IP. Confirm internet works. This is your control device during the whole process. | ☐ |
| 1.3 | Set Ring Camera 1 override | UniFi → Clients → Ring Camera 1 → gear icon → Virtual Network Override → VLAN 60 → Fixed IP 192.168.60.20 → Apply. | ☐ |
| 1.4 | Set Ring Camera 2 override | UniFi → Clients → Ring Camera 2 → gear icon → Virtual Network Override → VLAN 60 → Fixed IP 192.168.60.21 → Apply. | ☐ |
| 1.5 | Set SimpliSafe override | UniFi → Clients → SimpliSafe → gear icon → Virtual Network Override → VLAN 60 → Fixed IP 192.168.60.30 → Apply. | ☐ |
| 1.6 | Verify Security VLAN | Confirm Ring and SimpliSafe reappear in Clients with VLAN ID 60 and IPs in 192.168.60.x range. | ☐ |
| 1.7 | Set Sonos ZP 1 override | UniFi → Clients → Sonos ZP 1 → gear icon → Virtual Network Override → VLAN 30 → Fixed IP 192.168.30.10 → Apply. | ☐ |
| 1.8 | Set Sonos ZP 2 override | UniFi → Clients → Sonos ZP 2 → gear icon → Virtual Network Override → VLAN 30 → Fixed IP 192.168.30.11 → Apply. | ☐ |
| 1.9 | Set Printer Fixed IP | UniFi → Clients → HP Printer → gear icon → Fixed IP 192.168.50.50 → Apply. No VLAN override needed. | ☐ |
| 1.10 | Set Linux Box Fixed IP | UniFi → Clients → Linux Box → gear icon → Fixed IP 192.168.10.10 → Apply. No VLAN override needed. | ☐ |
| 1.11 | Set remaining IOT overrides | Work through all IOT devices in Section 5 of runbook — Virtual Network Override → VLAN 30. No Fixed IP needed for these. | ☐ |
| 1.12 | Connect kids devices to SandStorm | Confirm kids devices get 192.168.40.x IPs. Confirm curfew blocks at 10PM. | ☐ |
| 1.13 | Test Sonos from Main iPhone | Open Sonos app on iPhone — confirm speakers are visible. If not — check mDNS is ON on VLANs 10 and 30. | ☐ |
| 1.14 | Test printing from Main | Print test page from MacBook on Sassafrassss — confirm HP printer responds. | ☐ |
| 1.15 | Verify Ring app | Open Ring app — confirm cameras load and are accessible. | ☐ |
| 1.16 | Verify SimpliSafe app | Open SimpliSafe app — arm and disarm — confirm system responds. | ☐ |
| 1.17 | Check all Clients VLANs | UniFi → Clients → scan through all connected devices — confirm each shows correct VLAN ID. | ☐ |

⚠ NOTE: If a device is not showing in UniFi Clients — toggle WiFi off/on on the device. Give it 60 seconds.

# Phase 2 — Cutover

Only proceed here when every Phase 1 checkbox is ticked.

| Step | Action | Detail | ✓ |
| --- | --- | --- | --- |
| 2.1 | Confirm all Phase 1 steps complete | Do not proceed to cutover until all Phase 1 checkboxes are ticked and verified. | ☐ |
| 2.2 | Take pre-cutover EX7 backup | Settings → System → Backup → Download. Save as pre-cutover-live.unf to Google Drive. | ☐ |
| 2.3 | Enable AT&T IP Passthrough | Log into AT&T router → Firewall → IP Passthrough → Passthrough mode → select EX7 MAC address → Save. AT&T will reboot. | ☐ |
| 2.4 | Confirm EX7 gets public IP | EX7 → Settings → Internet → WAN IP should now show your public IP (not 192.168.x.x). May take 2–3 minutes after AT&T reboots. | ☐ |
| 2.5 | Confirm internet on Sassafrassss | Connect iPhone to Sassafrassss → browse — confirm internet works with public IP. | ☐ |
| 2.6 | Start 24hr burn-in | Leave everything running. Do not hide SSID yet. Let all devices reconnect naturally over the next 24 hours. | ☐ |

# Phase 3 — Post Burn-in (24 hours after Phase 2)

| Step | Action | Detail | ✓ |
| --- | --- | --- | --- |
| 3.1 | Audit UniFi Clients list | UniFi → Clients → confirm every expected device appears with correct VLAN and recent Last Seen timestamp. | ☐ |
| 3.2 | Cross-reference runbook Section 5 | Compare Clients list to MAC table — every device in Section 5 should appear. Investigate any missing devices. | ☐ |
| 3.3 | Disable Apple Private MAC | On every Apple device — disable Private Wi-Fi Address for Sassafrassss network. See Section 3 of runbook. | ☐ |
| 3.4 | Run all 21 verification tests | Work through Section 7 of runbook — check off every High priority test before proceeding. | ☐ |
| 3.5 | Hide Sassafrassss SSID | Settings → WiFi → Sassafrassss → enable Hidden → Save. | ☐ |
| 3.6 | Tighten to WPA3 | Settings → WiFi → Sassafrassss → Security → WPA3 only → Save. | ☐ |
| 3.7 | Decommission AT&T router | Unplug AT&T router. Confirm internet still works on EX7. Store AT&T router as backup. | ☐ |
| 3.8 | Final config backup | Settings → System → Backup → Download. Save as sassafrassss-final-backup.unf to Google Drive. | ☐ |

# Rollback Procedures

AT&T router is your safety net throughout this entire process. Do not decommission it until Phase 3 Step 3.7.

| Scenario | Action |
| --- | --- |
| Internet drops after passthrough | Log into AT&T router → disable IP Passthrough → reboot AT&T. EX7 returns to double NAT — internet restored. |
| Device not appearing in Clients | Check device is connected to Sassafrassss_Home. Try toggling WiFi off/on. If still missing — check MAC override is set correctly. |
| Ring/SimpliSafe offline | Confirm VLAN 60 override is set. Check Security_Isolate rule is not blocking outbound internet. Ping 8.8.8.8 from VLAN 60 subnet. |
| Sonos not visible | Confirm mDNS is ON on VLANs 10 and 30. Confirm IGMP Snooping is OFF on VLAN 30. Confirm Shared_Media firewall rule is above IOT_Isolate. |
| Need to fully rollback | Power off EX7. Rename AT&T SSID back to Sassafrassss. Reboot AT&T. All devices return to AT&T network automatically. |

# Quick Reference

Key IPs

- EX7 Management: 192.168.1.1

- AT&T Router: 192.168.1.254

- Sonos ZP 1: 192.168.30.10

- Sonos ZP 2: 192.168.30.11

- HP Printer: 192.168.50.50

- Ring Camera 1: 192.168.60.20

- Ring Camera 2: 192.168.60.21

- SimpliSafe: 192.168.60.30

- Linux Box: 192.168.10.10

VLAN Quick Reference

- VLAN 10 — Main (192.168.10.x) — Personal trusted devices

- VLAN 30 — IOT (192.168.30.x) — Appliances, lights, Sonos, Echos

- VLAN 40 — Restricted (192.168.40.x) — Kids devices

- VLAN 50 — Media (192.168.50.x) — Roku, Cast TV, Printer, WFH

- VLAN 60 — Security (192.168.60.x) — Ring cameras, SimpliSafe

If You Need Help

- Upload sassafrassss_runbook_v5.docx to a new Claude chat

- Starter prompt: I am configuring a UniFi EX7 home network. My network runbook is attached. Please read it and act as my senior WiFi network engineer. Ask me where I left off.